clc;

clear;

close all;

del_t = 0.4 * 10^-3;

n = linspace(-8, 7, 16);

sampled_time_v = [0.6875, 0.3261, 0.5429, 1.2022, ...

1.6875, 1.6327, 1.3384, 1.3655, 1.8125, 2.1739, 1.9571, ...

1.2978, 0.8125, 0.8673, 1.1616, 1.1345]; %9th sample is origin. array was missing 1.6327

L = length(sampled_time_v);

fqs = 1/del_t * 1/L * (-L/2:L/2-1); %herp derp i looked it up. PLEASE CHANGE THIS

figure;

plot(sampled_time_v)

%Testing update speed

fft = fft(fftshift(sampled_time_v)); %fftshift swaps left and right halves of vector. here, 9:16 get moved to front

figure;                              %thus array goes from 1:16 to 9:16then1:8

subplot(2,1,1)

plot(fqs, fftshift(abs(fft)))                     %SOMEONE LABEL THESE PLOTS PLZ

subplot(2,1,2)

plot(fqs, fftshift(angle(fft)))% this should be the end of question 1 (with some explanation)

clear all;

Part 2.

Notes: f(t) <-> f(w);

g(t)=f(-t) <-> G(w) =F(-w)

for DFT: f[n]: list of values at n=0, ending at N=N-1

f[n] = f_0d[n] +f_1d[n-1]+...+f_[N-1] d[n-(N-1)]

g[n] = f[-n] -> f_0d[-n] + f_1d[-n-1]mod(N)

g[n] = f_o

clear all;

Init Functions/Variables

fs = 32;

dt = 1/fs;

w_o = 2*pi;

A = 1;

N= 64; % length of sampled

n = (0:N-1)*dt;

w = (fs/N*(0:N-1));

%

fn = @(n) A*sin(w_o*n);% Normal samples from d[0]->d[n-(N-1)]

gn = flip(fn(n)); % Flip emulating -n

gn =circshift(gn,1); % Circular shift(mod) to f_o* d[n]

%In continuous g(t) <-> f(-t), due to

fw = fft(fn(n),N);

gw = fft(gn,N);

fw_n = fft(fn(-n),N);

wShift = fs/N*(-N/2:N/2-1); % Centering

Plots of Time Reversals

figure;

subplot(2,2,1);

plot(n,fn(-n), 'b');

hold on;

plot(n,fn(n),"--");

title("f[-nT] vs f[nT]")

xlabel("nT");

ylabel("Amplitude");

legend("f[-nT] Time Reversal", "f[nT] Original Signal")

subplot(2,2,2);

plot(n,gn, 'r');

title("g[nT] vs f[nT]");

xlabel("nT");

ylabel("Amplitude");

hold on;

plot(n,fn(n),"--");

legend("g[nT] Discrete time reversal", "f[nT] Original signal")

subplot(2,2,3)

plot(wShift, abs(fftshift(gw)));

title("G[w]")

xlabel("w");

ylabel("Amplitude");

xlim([-4,4]);

subplot(2,2,4)

plot(wShift, abs(fftshift(fw_n)));

title("F[-w]");

xlabel("w");

ylabel("Amplitude");

xlim([-4,4]);

Error

e_gw = rmse(fw_n,gw);

e_s = rmse(fn(-n),gn);

figure;

dif= abs(fw_n-gw);

plot(wShift,fftshift(dif));%Maximal difference at w_o/2

title("Difference F[-w] and G[w]")

ylabel("Absolute Amplitude")

This should be good enough for pt. 2

the error between the F[-M] and G[M] is in the pico units so I would say negligable. The highest peak in error though is around the w_o/2

Part 3.

%will generate two 1-D signals, then combine to a 2-D signal, then do 2D

%fft and two 1d fft and compare

%will generate a white gaussian and a sinusoid, each composed of 64 points from

baseValues = linspace(-2*pi, 2*pi, 128); % generating base signal from -2pi to 2pi. will use center -pi to pi portion

delta_t = baseValues(1)-baseValues(2);  % record sample rate

g_x = 1/sqrt(2*pi)*exp(-0.5*baseValues.^2); %generate & plot gaussian curve from -2pi to 2pi

subplot(8,4,1)

plot(baseValues,g_x)

xlabel('t')

title('g_x')

subplot(8,4,3)

plot(baseValues(33:96), g_x(33:96)) %plot gaussian curve from -pi to pi

xlabel('t')

title('g_x base')

g_y = cos(baseValues);  %generate and plot cosine from -2pi to 2pi

subplot(8,4,2)

plot(baseValues,g_y)

xlabel('t')

title('g_y')

subplot(8,4,4)

plot(baseValues(33:96), g_y(33:96)) %plot cosine from -pi to pi

xlabel('t')

title('g_y base')

[tempX, tempY] = meshgrid(g_x, g_y);                    %generate fully sized array of X and Y to prep for 3D

[tempXX, tempYY] = meshgrid(g_x(33:96), g_y(33:96));    %same as above but limited to -pi to pi

g_f = tempX.*tempY;     % generate 2d plots for both limits and plot in 3d and in heat map for visualization

g_ff = tempXX.*tempYY;  % plotted both -2pi to 2pi and -pi to pi plots to imply the signals are repeating

subplot(8,4,[5,6,9,10]) % and just a window of the repeating portion is being observed

surf(baseValues, baseValues, g_f)

subplot(8,4,[13,14,17,18])

imagesc(baseValues, baseValues, g_f)

subplot(8,4,[7,8,11,12])

surf(baseValues(33:96), baseValues(33:96), g_ff)

subplot(8,4,[15,16,19,20])

imagesc(baseValues(33:96), baseValues(33:96), g_ff)

G_x = fft(fftshift(g_x(33:96)));    % generate the 1D ffts using fftshift to maintin zero

G_y = fft(fftshift(g_y(33:96)));    % position from the center of the array

subplot(8,4,21)

plot(ifftshift(abs(G_x)))           % place ifftshift on frequency domain to show negative to positive freq

subplot(8,4,22)                     % (mostly to show gaussian in, gaussian out)

plot(ifftshift(abs(G_y)))

subplot(8,4,23)

plot(ifftshift(angle(G_x)))

subplot(8,4,24)

plot(ifftshift(angle(G_y)))

[fXmag,fYmag] = meshgrid(ifftshift(abs(G_x)), ifftshift(abs(G_y)));

G_f_mag = fXmag.*fYmag;     %repeat process from start of problem to generate 2D fft as the 1D fft's multiplied

[fXang,fYang] = meshgrid(ifftshift(angle(G_x)), ifftshift(angle(G_y)));

G_f_angle = fXang.*fYang;

subplot(8,4,[25,26])

surf(G_f_mag)

subplot(8,4,[27,28])

surf(G_f_angle)

G_ff = fft2(g_ff);      %generate 2D fft directly from the 2D time domain plot for comparison

subplot(8,4,[29,30])

surf(fftshift(abs(G_ff)))

title('2-D fft Mag')

subplot(8,4,[31,32])

surf(fftshift(angle(G_ff)))

title('2-D fft ang')

Part 4.

%Load and display grayscale 'Lena' image file

img = rescale(im2gray(imread('img_gray.png')));

imshow(img)

%Remove first row and column of img to create an array with odd dimensions

%Going from 512 x 512 to 511 x 511

img(1,:) = [];

img(:,1) = [];

%Pad image with zeros to produce new image of dimension (2^p + 1) x (2^p + 1)

%In this case, p=10 and padded img is 1025 x 1025

padding = ((2^10) + 1 - size(img,1))/2;

img_pad = padarray(img, [padding, padding]);

%Display padded image

imshow(img_pad)

%Set rotation angle step to 1 degree

del_phi = 1;

%Define projection matrix

p_phi = [];

%Simulate projections by rotating img and summing down columns

for phi_temp = 0:del_phi:179

img_temp = imrotate(img_pad, phi_temp, 'crop');

p_phi(phi_temp+1, :) = sum(img_temp);

end

%Display normalized projection matrix (sinogram)

imshow(p_phi./max(p_phi))

%Remove last column of proj mtx so 2nd dimension = 2^10

p_phi(:,2^10 + 1) = [];

%Perform fftshift followed by fft across the rows (central slices)

p_phi = fftshift(p_phi, 2);

P_phi = fft(p_phi,[],2);

P_phi = fftshift(P_phi, 2);

%Display normalized central slice matrix

imshow(abs(P_phi)./max(abs(P_phi)))

%Create arrays for use in griddata

phi = [0:1:179]';

omega_x_prime = [-size(P_phi,2)/2 : 1 : size(P_phi,2)/2 - 1];

omega_x_known = cosd(phi)*omega_x_prime;

omega_y_known = sind(phi)*omega_x_prime;

values_known = P_phi;

omega_x_target = ones(size(omega_x_prime,1),1)'*omega_x_prime;

omega_y_target = omega_x_target';

%Estimate F with linear interpolation of central slices

F_lin = griddata(omega_x_known,omega_y_known,values_known,omega_x_target,omega_y_target,'linear');

%Replace NaN values with 0

NaNs = isnan(F_lin);

F_lin(NaNs) = 0;

%Estimate F with nearest neighbor interpolation of central slices

F_near = griddata(omega_x_known,omega_y_known,values_known,omega_x_target,omega_y_target,'nearest');

%Replace NaN values with 0

NaNs = isnan(F_near);

F_near(NaNs) = 0;

%Estimate F with cubic interpolation of central slices

F_cub = griddata(omega_x_known,omega_y_known,values_known,omega_x_target,omega_y_target,'cubic');

%Replace NaN values with 0

NaNs = isnan(F_cub);

F_cub(NaNs) = 0;

%Display real part of F w/ linear interpolation

imshow(F_lin)

%Estimate f using 2D ifft of F for each interpolation type

f_lin = fftshift(ifft2(fftshift(F_lin)));

f_near = fftshift(ifft2(fftshift(F_near)));

f_cub = fftshift(ifft2(fftshift(F_cub)));

%Display magnitude of f w/ linear interpolation

imshow(rescale(abs(f_lin)))

%Display magnitude of f w/ nearest neighbor interpolation

imshow(rescale(abs(f_near)))

%Display magnitude of f w/ linear interpolation

imshow(rescale(abs(f_cub)))

%Remove 1st row and column of zeros in original padded image

%so the dimensions will match with reconstructed images

img_pad(1,:) = [];

img_pad(:,1) = [];

%Calculate error magnitude matrix of reconstructed images for each interpolation type

err_lin = double(img_pad) - abs(f_lin);

err_cub = double(img_pad) - abs(f_cub);

err_near = double(img_pad) - abs(f_near);

%Display error magnitude plots for each interpolation type

imshow(err_lin)

imshow(err_cub)

imshow(err_near)

%Calculate mean squared error (MSE) between each reconstruction and original img

mse_lin = immse(double(img_pad), f_lin);

mse_cub = immse(double(img_pad), f_cub);

mse_near = immse(double(img_pad), f_near);

%Print MSEs

fprintf('\n The mean-squared error for linear interpolation is %0.4f\n \n The mean-squared error for cubic interpolation is %0.4f\n \n The mean-squared error for nearest neighbor interpolation is %0.4f\n', mse_lin, mse_cub, mse_near);